|  |  |  |  |  | Reg. No.: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Name : |  |  |  |  |  |  |
| Continuous Assessment Test II – October-2024 |  |  |  |  |  |  |  |  |  |  |  |
| Programme |  |  | : | B.Tech. (ECE/ECM) |  | Semester |  | : | FS 2024-25 |  |  |
| Course |  |  | : | Embedded C Programming |  | Code |  | : | BECE320E |  |  |
|  |  |  |  |  |  | Class Nbr |  | : | CH2024250102674 |  |  |
| Faculty |  |  | : | Prof. Srinivasan R |  | Slot |  | : | E1 |  |  |
| Time |  |  | : | 90 Minutes |  | Max. Marks |  | : | 50 |  |  |
| Answer ALL the questions. |  |  |  |  |  |  |  |  |  |  |  |
| Q. No. | Sub Sec. | Questions |  |  |  |  |  |  |  | Marks | BT Level |
| 1. |  | Find the hex values to be filled into the appropriate special function registers of 8051 for the following tasks. - Generation of 1 kHz clock signal using timer 0 in mode 1. Assume the crystal frequency as 12 MHz. - Serial port communication with receiving enabled in mode 3 at a baud rate of 19,200. Assume the crystal frequency as 11.0592 MHz. - Enabling serial, timer 1, and EXT 0 interrupts and assigning high priority to timer 1 interrupt. - Counting of external pulses by timer 0 in mode 2. |  |  |  |  |  |  |  | 12 | L2 |
| 2. |  | In an 8051-based system, there is a need to store additional program code by expanding the memory. If 32 kB memory is to be interfaced, sketch the interface between the 8051 and the memory. |  |  |  |  |  |  |  | 6 | L1 |
| 3. |  | Write an 8051 C program to implement a digital safe. The correct combination to unlock the safe is 0110, entered through switches connected to P1.0 to P1.3. When the correct combination is entered, set P3.0 high to unlock the safe and trigger an LED connected to P3.1 for 10 seconds to indicate the safe is unlocked. Use ‘for’ loops to generate delay. Assume the crystal frequency as 11.0592 MHz. |  |  |  |  |  |  |  | 8 | L3 |
| 4. |  | Write an embedded C program for an 8051-based water level monitoring system. A water level sensor, interfaced with port P2, provides an 8-bit digital output representing the water level (range: 0-255). An alarm buzzer connected to pin P1.5 is activated when the water level exceeds a predefined threshold of 80 and deactivated once the level drops below the threshold. |  |  |  |  |  |  |  | 6 | L3 |
| 5. |  | Write an 8051 C program to convert packed BCD 0x47 to ASCII and display the bytes on P2 and P0. |  |  |  |  |  |  |  | 5 | L1 |
| 6. |  | Write an 8051 C program to send out the value 23H serially, one bit at a time, via P1.2. The MSB should go out first. |  |  |  |  |  |  |  | 5 | L2 |
| 7. |  | Write an 8051 embedded C program to generate a square wave of 500 Hz frequency on pin P1.3. Use timer 1 in mode 1. Assume crystal frequency as 16 MHz. |  |  |  |  |  |  |  | 8 | L3 |

óóó